Welding Workstation Safety Instruction (SOP)

Based on regulatory requirements and best practices for safe welding operations

Purpose

This document defines the safety requirements, equipment standards, and operational procedures for a professional welding workstation. Its goal is to ensure safe, ergonomic, and compliant welding activities in accordance with applicable regulations and technical norms.

2. Scope

This instruction applies to all employees performing welding or metal cutting tasks at designated welding stations equipped with welding tables, ventilation systems, and personal protective equipment (PPE).

3. Responsibilities

Welders

- Follow all safety rules and workstation procedures.

- Use required PPE at all times.

- Inspect equipment before use and report defects immediately.

Supervisors

- Ensure workstations meet technical and organizational safety requirements.

- Provide training and maintain documentation.

- Verify proper functioning of ventilation, grounding, and fire protection systems.

4. Required Personal Protective Equipment (PPE)

Employees must wear the following PPE before starting any welding activity:

- Welding helmet – protects eyes and face from arc radiation, sparks, and metal spatter.

- Flame-resistant protective clothing – made of heat-resistant materials such as leather or technical fabrics.

- Welding gloves – protect hands from burns and mechanical injuries.

- Safety footwear – with anti-slip soles and resistance to hot particles.

- Respiratory protection – required in poorly ventilated areas or when using electrodes generating heavy fumes.

5. Workstation Requirements

5.1 Ventilation and Fume Extraction

- Each welding station must be equipped with local fume extraction or general ventilation.

- Mobile extraction units may be used with system tables to remove fumes through tabletop openings.

- Ventilation must be checked regularly for proper operation.

5.2 Workspace Organization

- Keep the area around the welding table free from flammable or explosive materials.

- Arrange cables, tools, and hoses to prevent damage, tripping, or entanglement.

- Maintain clear access to emergency exits.

5.3 Fire Protection

- A functional fire extinguisher must be located near the workstation.

- Fire response instructions must be visible and accessible.

- Welding curtains should be used in larger facilities to protect bystanders from arc radiation.

6. Welding Table Requirements

6.1 Structural Stability

- The welding table must remain rigid under heavy loads.

- Internal steel reinforcement and adjustable legs or swivel feet must ensure stability and proper leveling.

6.2 Grounding

- The table must allow connection to a grounding system to prevent uncontrolled current flow and reduce electric shock risk.

6.3 Work Surface Material

- The tabletop must be made of heat-resistant steel with an oxide layer protecting against corrosion and spatter.

- Only compatible accessories (e.g., D16 or D28 system components) may be used.

7. Ergonomics and Accessories

- The workstation must support ergonomic posture and reduce strain injuries.

- Height-adjustable tables should be adapted to the operator’s height and task type.

- System accessories (towers, extensions, clamps) must be used for safe and precise positioning of workpieces.

8. Operational Procedure

Before Starting Work

- Inspect the welding table, equipment, and accessories for damage.

- Verify proper grounding.

- Ensure the tabletop is clean and free of slag or spatter.

- Check ventilation and extraction systems.

- Put on all required PPE.

During Work

- Maintain a clean and organized workspace.

- Use only approved accessories compatible with the workstation system.

- Ensure adequate lighting.

- Monitor ventilation performance.

After Completing Work

- Turn off welding equipment and ventilation.

- Clean the tabletop and remove slag or debris.

- Store PPE in a clean, dry place.

- Report any equipment issues to the supervisor.

9. Safety Principles

Safe welding results from the combination of:

- Knowledge of regulations

- Proper habits and safe behavior

- High-quality equipment and workstation design

A professional welding table is a key element ensuring precision, ergonomics, and safety during daily welding operations.